These settings configure seamless roaming between multiple EKI6333AC-2GD access points.

Important: Roaming works when the access points are connected to each other via Ethernet (wired backhaul).

To configure each access point:

- Connect a computer directly to the AP using an Ethernet cable.

- Configure each AP individually using the settings below.

| Settings | AP1 | AP2 (and any other APs) |
| --- | --- | --- |
| Basic |  |  |
| - Operation Mode | Access Point | Access Point |
| - 2G SSID | AMR-AP-2G | AMR-AP-2G |
| - 5G SSID | AMR-AP-5G | AMR-AP-5G |
| - Channel | Any | Any (different from AP1) |
| Advanced |  |  |
| - Transmission Power | Change for signal strength desired | Change for signal strength desired |
| Security |  |  |
| - Security Mode | WPA-Personal | WPA-Personal |
| - WPA Version | WPA2 | WPA2 |
| - WPA Cipher | TKIP+AES | TKIP+AES |
| - Pass Phrase | Same | Same |
| - 802.11r | Enabled | Enabled |
| - NAS ID | Same | Same |
| - Mobility Domain | Same | Same |
| LAN |  |  |
| - Network Mode | Static | Static |
| - LAN IP | 192.168.1.2 | 192.168.1.3 |
| - Subnet Mask | 255.255.255.0 | 255.255.255.0 |
| - Gateway | None | None |
| - DNS | None | None |
| - DHCP Server | Enabled | Disabled |
| - Start IP | 192.168.1.100 | None |

Notes

- Use the same SSIDs, security settings, NAS ID, and Mobility Domain on every AP.

- Assign each AP a unique static LAN IP address.

- Configure different Wi-Fi channels on neighboring APs to reduce interference.

- Adjust transmit power as needed to provide the desired overlap between AP coverage areas for smooth roaming.

- Do not set network mode to DHCP. This changes the AP’s IP address to that of the router it is connected to.  If no router is connected, it will not receive an IP address and will need to be reset in order to access.

(no router -> no ip -> cannot access -> hard reset)